| Credit Requirement: |  | a) Provision of Acoustic Treatment Maximum 5 credits for providing the following listed acoustic treatment: i. Air-cooled Chiller – Erect a barrier/ install silencer for air-cool chiller; ii. Water-cooled Chiller – Being enclosed in an acoustic enclosure or plantroom; iii. Cooling Tower – Erect a barrier/ install silencer for cooling tower; iv. Fan – Installation of flexible connector; v. Fan (for sound power level > 80dB(A)) – Provide silencers at major fan discharge outlets (for exhaust fans) or at air inlets (for intake fans); vi. Fan – Reduce the speed of fans at non-rushed hours; vii. Air duct – Stiffen the vibrating duct surface with supporting webs; viii. Air duct – Apply damping material to the vibrating duct surface; ix. Air duct – Apply composite lagging of sound absorbing materials; x. Chiller pumps – Erect a barrier/ located indoor; and xi. Water pumps – Erect a barrier/ located indoor. b) Demonstration of Compliance with HKPSG Criteria 1 credit for demonstrating that the level of the intruding noise at the façade of the potential Noise Sensitive Receivers (NSRs) is in compliance with the criteria recommended in the Hong Kong Planning Standards and Guidelines (HKPSG). |  |  |
| --- | --- | --- | --- | --- |
| Credit Attainable: |  |  | 6 |  |
| Credit(s) anticipated for this submission: |  |  |  |  |
| Explanation to fulfill BEAM Plus criteria: |  |  |  |  |
| Enclosures: |  |  |  |  |
| a) Provision of Acoustic Treatment |  |  |  |  |
| ☐ | Equipment catalogues with sound power level indicated, operation schedule, drawings showing the provision of acoustic treatment for chillers, cooling towers, ventilation fans, air ducts, pumps. |  |  |  |
| ☐ | Record photographs of the acoustic treatment. |  |  |  |
| ☐ | Other supporting documents, please specify. |  |  |  |
| b) Demonstration of Compliance with HKPSG Criteria |  |  |  |  |
| ☐ | Summary table listing the nearest NSRs, building equipment sound level, quantities, ANL and noise level at the façade of the nearest NSRs. |  |  |  |
| ☐ | Location plan to indicate the positions of the NSRs and building equipment. |  |  |  |
| ☐ | Equipment catalogues. |  |  |  |
| ☐ | Calculation or measurement. |  |  |  |
| ☐ | Other supporting documents, please specify. |  |  |  |