| Credit Requirement: |  |  |  | 6 credits if there are no external lightings installed for the building. External lightings include advertisement board, façade lighting and video walls. Alternatively Maximum 6 credits for implementing the following listed features: i. Provide automatic control (e.g. timer switch) to switch off the external lighting (23:00 to 07:00 hours); ii. Avoid over-illumination of signs, facades, shop fronts, video walls and facilities with lighting. Over-illumination will increase possibility of light pollution; iii. Position and aim the lightings properly to avoid overspill of light to outside the area being lit up; iv. Use lightings with appropriate shields, baffles, louvers and cut-off features to prevent light overspill to nearby residence and into the sky, and glare from the light source; v. Circulate the Guidelines on Industry Best Practices for External Lighting Installations to building users; vi. Switch off all external lightings from the Building Owners/ Building Management Company (23:00 to 07:00 hours); and vii. Switch off all external lightings from all building users (23:00 to 07:00 hours). |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Credit Attainable: |  |  |  |  | 6 |  |
| Credit(s) anticipated for this submission: |  |  |  |  |  |  |
| Checklist: |  |  |  |  |  |  |
| ☐ |  | There are no external lightings installed for the building |  |  |  |  |
| Alternatively |  |  |  |  |  |  |
| ☐ |  | The following feature(s) is/are implemented: |  |  |  |  |
|  |  | ☐ | Provide automatic control (e.g. timer switch) to switch off the external lighting (23:00 to 07:00 hours) |  |  |  |
|  |  | ☐ | Avoid over-illumination of signs, facades, shop fronts, video walls and facilities with lighting. Over-illumination will increase possibility of light pollution. |  |  |  |
|  |  | ☐ | Position and aim the lightings properly to avoid overspill of light to outside the area being lit up |  |  |  |
|  |  | ☐ | Use lightings with appropriate shields, baffles, louvers and cut-off features to prevent light overspill to nearby residence and into the sky, and glare from the light source. |  |  |  |
|  |  | ☐ | Circulate the Guidelines on Industry Best Practices for External Lighting Installations to building users. |  |  |  |
|  |  | ☐ | Switch off all external lightings from the Building Owners/ Building Management Company (23:00 to 07:00 hours). |  |  |  |
|  |  | ☐ | Switch off all external lightings from all building users (23:00 to 07:00 hours). |  |  |  |
| Explanation to fulfill BEAM Plus criteria: |  |  |  |  |  |  |
| Enclosures: |  |  |  |  |  |  |
| ☐ | Record photographs of external area and exterior of the building. |  |  |  |  |  |
| ☐ | Layout/ building services drawings demonstrating that there are no external lightings installed for the building. |  |  |  |  |  |
| ☐ | Other supporting documents, please specify. |  |  |  |  |  |
| Alternatively, |  |  |  |  |  |  |
| ☐ | Narrative of the strategies and the combination (if any). |  |  |  |  |  |
| ☐ | Summary table listing quantity and operation schedule of all external lightings. |  |  |  |  |  |
| ☐ | Control schematic diagram and electrical wiring diagram, showing provision of automatic control. |  |  |  |  |  |
| ☐ | Location plan to indicate the external lightings. |  |  |  |  |  |
| ☐ | External light management policy endorsed by top management. |  |  |  |  |  |
| ☐ | Record photographs (before and after the switch off of external lightings and lighting with appropriate shields, baffles, louvers and cut-off features). |  |  |  |  |  |
| ☐ | Acknowledgement receipt from building users for receiving the Guidelines on Industry Best Practices for External Lighting Installations. |  |  |  |  |  |
| ☐ | Other supporting documents, please specify. |  |  |  |  |  |