| Credit Requirement: |  |  | Maximum 5 credits for implementing the following energy saving practices: i. Turn on equipment/ systems based on operational hours of buildings. (Operation schedule); ii. Avoid pre-cooling. Switch on centralised A/C system not more than 30 minutes in advance in the morning. (Operation schedules of AC and building); iii. For premises where the A/C systems are provided with heaters, avoid operating the heaters when the outdoor air temperature is above 20°C. (Operation schedule and/or print screen of BMS showing temperature setting); iv. Turn off lighting if it is not needed. (Operation schedule and/or photograph showing timer setting); v. Cut down number of lamps/ luminaires in area over-lit (over CIBSE recommendation) by artificial lighting and in perimeter area sufficiently lit by natural daylight. (Photographs showing lux measurement and de-lamping); vi. Encourage using the stairs (for one or two floors up or down) rather than taking the lift. (Site photograph showing notice/ poster to encourage tenant/ building user); vii. Shut down some of the lifts and escalators during non-peak hours. (Operation schedule); and viii. Arrange routine cleaning schedule for light diffusers, globes and reflectors to ensure light output efficiency (Cleaning schedule). |  |  |
| --- | --- | --- | --- | --- | --- |
| Credit Attainable: |  |  |  | 25 |  |
| Credit(s) anticipated for this submission: |  |  |  |  |  |
| Checklist: |  |  |  |  |  |
| The following energy saving practice(s) is/are implemented: |  |  |  |  |  |
| ☐ |  | Turn on equipment/ systems based on operational hours of buildings. (Operation schedule) |  |  |  |
| ☐ |  | Avoid pre-cooling. Switch on centralised A/C system not more than 30 minutes in advance in the morning. (Operation schedules of AC and building) |  |  |  |
| ☐ |  | For premises where the A/C systems are provided with heaters, avoid operating the heaters when the outdoor air temperature is above 20°C. (Operation schedule and/or print screen of BMS showing temperature setting) |  |  |  |
| ☐ |  | Turn off lighting if it is not needed. (Operation schedule and/or photograph showing timer setting) |  |  |  |
| ☐ |  | Cut down number of lamps/ luminaires in area over-lit (over CIBSE recommendation) by artificial lighting and in perimeter area sufficiently lit by natural daylight. (Photographs showing lux measurement and de-lamping) |  |  |  |
| ☐ |  | Encourage using the stairs (for one or two floors up or down) rather than taking the lift. (Site photograph showing notice/ poster to encourage tenant/ building user) |  |  |  |
| ☐ |  | Shut down some of the lifts and escalators during non-peak hours. (Operation schedule) |  |  |  |
| ☐ |  | Arrange routine cleaning schedule for light diffusers, globes and reflectors to ensure light output efficiency (Cleaning schedule) |  |  |  |
| Explanation to fulfill BEAM Plus criteria: |  |  |  |  |  |
| Enclosures: |  |  |  |  |  |
| ☐ | A short report on the narrative of the practices and measures. |  |  |  |  |
| ☐ | Other supporting documents, please specify. |  |  |  |  |