| Project Name: |  | CIR Applied: | Y | N |
| --- | --- | --- | --- | --- |
| Project Code: |  | CIR Number: |  |  |

| Extent of Application: | All Space Types |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Objectives: | Interior layout provisions that embrace healthiness and safety. |  |  |  |  |
| Credit Requirement: | 1 to 2 credit points for scoring at least 50%/ 70% of the applicable OHS measures and facilities within the assessment boundary. Alternatively, - 2 credit points will be awarded if the assessment boundary has been certified with ISO 45001 certification. |  |  |  |  |
| Credit Point(s) Attainable: |  | 2 |  |  |  |
| Credit Point(s) anticipated for this submission: |  |  | 1 |  | 2 |

| COMPLIANCE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | The following Occupational Health and Safety (OHS) measures and/ or facilities are provided: |  |  |  |  |  |  |  |  |  |  |
|  | Ergonomics |  |  |  |  |  |  |  |  |  |  |
|  |  | Height-adjustable desks [10% of the workstations] |  |  |  |  |  | Headsets [10% of the workstations] |  |  |  |
|  |  | “Ergonomic” mice [100% of the workstations] |  |  |  |  |  | Wrist rests [10% of the workstations] |  |  |  |
|  |  | Sufficient space below desks [100% of the workstations] |  |  |  |  |  | Stretching equipment |  |  |  |
|  |  | Desk/ tables are rounded corners with no sharp edges [100% of the workstations] |  |  |  |  |  |  |  |  |  |
|  |  | Anti-fatigue matting/ footrests [10% of the workstations] |  |  |  |  |  |  |  |  |  |
|  |  | Adjustable chairs [100% of the workstations] |  |  |  |  |  |  |  |  |  |
|  |  | Rotary chairs with five-pronged base [100% of the workstations] |  |  |  |  |  |  |  |  |  |
|  |  | Height-adjustable stands for monitor [50% of the workstations] |  |  |  |  |  |  |  |  |  |
|  |  | Monitors with built-in height adjustment [100% of the workstations] |  |  |  |  |  |  |  |  |  |
|  |  | Others as proposed by the Applicant: (Please specify) |  |  |  |  |  |  |  |  |  |
|  | Storage |  |  |  |  |  |  |  |  |  |  |
|  |  | Closed lid bins for all waste storage |  |  |  |  |  |  |  |  |  |
|  |  | A distance of 1.2m in front of storage to provide sufficient space for safe movement |  |  |  |  |  |  |  |  |  |
|  |  | Safe means of climbing up to storage with a height more than 2m |  |  |  |  |  |  |  |  |  |
|  |  | Others as proposed by the Applicant: (Please specify) |  |  |  |  |  |  |  |  |  |
|  | Interior Layout |  |  |  |  |  |  |  |  |  |  |
|  |  | No exposed electrical extension cords crossing walkway/ corridor |  |  |  |  |  |  |  |  |  |
|  |  | No materials that can easily catch fire left near a heat source |  |  |  |  |  |  |  |  |  |
|  |  | Others as proposed by the Applicant: (Please specify) |  |  |  |  |  |  |  |  |  |
|  | Summary |  |  |  |  |  |  |  |  |  |  |
|  | Number of measures and/ or facilities provided: |  |  |  |  | (Total claimed items) / |  |  |  | (Total applicable items) |  |
|  | Percentage of measures and/ or facilities provided: |  |  |  |  | % |  |  |  |  |  |
|  | Please provide justification for any non-applicable sub-item below: |  |  |  |  |  |  |  |  |  |  |
|  | Certified with ISO 45001 certification |  |  |  |  |  |  |  |  |  |  |
|  | Date of Certification: |  |  |  |  |  |  |  |  |  |  |
|  |  | Corresponding actions are implemented in accordance with ISO 45001 |  |  |  |  |  |  |  |  |  |

SUBMITTALS

| Supporting Documents Please provide softcopies with filename prefix as indicated in the leftmost column below. |  |  |
| --- | --- | --- |
| MAN-04-04_00 | BEAM Plus Interiors submission form for MAN-04-04 | ¨ |
| MAN-04-04_01 | Summary on OHS measures and facilities |  |
| MAN-04-04_02 | Photo record(s) |  |
| MAN-04-04_03 | ISO 45001 certificate |  |

SPECIAL CIRCUMSTANCE

Describe the circumstance limiting the project team’s ability to complete this form or provide the submittals required in this template. Be sure to reference what additional documentation has been provided, if any. Non-standard documentation will be considered upon its merits (Optional)