| Project Name: |  | CIR Applied: | Y  | N |
| --- | --- | --- | --- | --- |
| Project Code: |  | CIR Number: |  |  |

| Extent of Application: | All Space Types |
| --- | --- |
| Objectives: | Encourage implementation of germ-resistance management to keep the interior spaces clean and reduce potential spread of germ. |
| Credit Requirement: | - Moisture Control 1 credit point for implementing measures to reduce the potential of moisture occurrence and accumulation. - Antimicrobial High Touch Surface 1 credit point if all high touch surfaces are coated with or comprised of or sprayed with material that is abrasion-resistant, non-leaching and antimicrobial like copper, brass and plexiglass. - Anti-microbial Wall Surface 1 credit point if at least 50% of the wall surface has applied antimicrobial paint or coating. Percentage of the application of antimicrobial wall surface (%) should be calculated by below equation: Application of Antimicrobial Wall Surface % =Wall Surface With Antimicrobial Paint or Coating Wall Surface×100% |

| Credit Point(s) Attainable: | 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Credit Point(s) anticipated for this submission: |  | 1 |  | 2 |  | 3 |

| COMPLIANCE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (a) Moisture Control |  |  |  |  |  |  |  |
|  | Moisture management measures are effectively implemented |  |  |  |  |  |  |
|  | The following measures are adopted: |  |  |  |  |  |  |
|  |  | Investigation and inspection for moisture control |  |  |  |  |  |
|  |  | Use moisture tolerant materials |  |  |  |  |  |
|  |  | Additional or alternative measure(s) proposed (Please specify with substantiation): |  |  |  |  |  |
|  | The technical report provided covers the following items: |  |  |  |  |  |  |
|  |  | List of each adopted measures for moisture management |  |  |  |  |  |
|  |  | Detailed description of each adopted measures and explanation on how it could prevent condensation on cold surfaces |  |  |  |  |  |
|  |  | Evidence showing the adoption of measures |  |  |  |  |  |
| (b) Antimicrobial High Touch Surface |  |  |  |  |  |  |  |
|  | All high touch surfaces within access route of the assessment boundary are coated with/ comprised of/ sprayed with material that is abrasion-resistant, non-leaching and antimicrobial |  |  |  |  |  |  |
|  | The technical report covers the following items: |  |  |  |  |  |  |
|  |  | Layout plan identifying the concerned high touch surfaces within the access route of the assessment boundary |  |  |  |  |  |
|  |  | Evidence showing the adoption of measures |  |  |  |  |  |
| (c) Antimicrobial Wall Surface |  |  |  |  |  |  |  |
|  | Total area of wall surface: |  |  | Total area of wall surface with antimicrobial paint or coating: |  |  |  |
|  | Percentage of the wall surface that applied antimicrobial paint or coating is: |  |  |  |  | % |  |
|  | Extent of wall surface with anti-microbial paint or coating: |  |  |  |  |  |  |
|  | Evidence showing the adoption antimicrobial paint or coating |  |  |  |  |  |  |

SUBMITTALS

| Supporting Documents Please provide softcopies with filename prefix as indicated in the leftmost column below. |  |  |
| --- | --- | --- |
| - Moisture Control |  |  |
| MW-01-06a_00 | BEAM Plus Interiors submission form for MW-01-06a |  |
| MW-01-06a_01 | A narrative on the design measures for moisture control |  |
| MW-01-06a_02 | A technical report on moisture control |  |
| - Antimicrobial High Touch Surface |  |  |
| MW-01-06b_00 | BEAM Plus Interiors submission form for MW-01-06b |  |
| MW-01-06b_01 | A technical report on the application of antimicrobial high touch surface |  |
| - Antimicrobial Wall Surface |  |  |
| MW-01-06c_00 | BEAM Plus Interiors submission form for MW-01-06c |  |
| MW-01-06c_01 | Calculation showing the percentage of wall surface area with antimicrobial paint or coating |  |
| MW-01-06c_02 | Supporting document including catalogue, valid test reports of the antimicrobial paint or coating and dated photo records for the application |  |

SPECIAL CIRCUMSTANCE

Describe the circumstance limiting the project team’s ability to complete this form or provide the submittals required in this template. Be sure to reference what additional documentation has been provided, if any. Non-standard documentation will be considered upon its merits (Optional)