| Project Name: |  |  |  |  |  |  | Project Code: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Log Period: (yyyy/mm) |  |  | to |  |  |  | Construction Stage |  |  |  |  | Demolition |
|  |  | Note: Each Log Sheet should not cover period longer than 3 months. |  |  |  |  |  |  |  |  |  | Foundation |
|  |  |  |  |  |  |  |  |  |  |  |  | Superstructure |
| IDCM-02-02c - Proactive wastewater management provisions to minimize water pollution during construction. |  |  |  |  | Properly carried out? |  |  |  | Rectifying Measures Required? |  |  |  |
|  | Surface Run-off |  |  |  | Yes | No |  | N/A | Yes |  |  |  |
|  | Surface run-off from construction sites should be discharged into storm drains via adequately designed sand/silt removal facilities such as sand traps, silt traps and sediment basins. Channels or earth bunds or sand bag barriers should be provided on site to properly direct stormwater to such silt removal facilities. Perimeter channels at site boundaries should be provided where necessary to intercept storm run-off from outside the site so that it will not wash across the site. Catchpits and perimeter channels should be constructed in advance of site formation works and earthworks. |  |  |  |  |  |  |  |  |  |  |  |
|  | Silt removal facilities, channels and manholes should be maintained and the deposited silt and grit should be removed regularly, at the onset of and after each rainstorm to ensure that these facilities are functioning properly at all times. |  |  |  |  |  |  |  |  |  |  |  |
|  | Construction works should be programmed to minimize soil excavation works in rainy seasons (April to September). If excavation in soil could not be avoided in these months or at any time of year when rainstorms are likely, for the purpose of preventing soil erosion, temporarily exposed slope surfaces should be covered e.g. by tarpaulin, and temporary access roads should be protected by crushed stone or gravel, as excavation proceeds. Intercepting channels should be provided (e.g. along the crest/edge of excavation) to prevent storm runoff from washing across exposed soil surfaces. Arrangements should always be in place to ensure that adequate surface protection measures can be safely carried out well before the arrival of a rainstorm. |  |  |  |  |  |  |  |  |  |  |  |
|  | Earthworks final surfaces should be well compacted and the subsequent permanent work or surface protection should be carried out immediately after the final surfaces are formed to prevent erosion caused by rainstorms. Appropriate drainage like intercepting channels should be provided where necessary. |  |  |  |  |  |  |  |  |  |  |  |
|  | Measures should be taken to minimize the ingress of rainwater into trenches. If excavation of trenches in wet seasons is necessary, they should be dug and backfilled in short sections. Rainwater pumped out from trenches or foundation excavations should be discharged into storm drains via silt removal facilities. |  |  |  |  |  |  |  |  |  |  |  |
|  | Open stockpiles of construction materials (e.g. aggregates, sand and fill material) on sites should be covered with tarpaulin or similar fabric during rainstorms. Measures should be taken to prevent the washing away of construction materials, soil, silt or debris into any drainage system. |  |  |  |  |  |  |  |  |  |  |  |
|  | Manholes (including newly constructed ones) should always be adequately covered and temporarily sealed so as to prevent silt, construction materials or debris from getting into the drainage system, and to prevent storm run-off from getting into foul sewers. Discharge of surface run-off into foul sewers must always be prevented in order not to unduly overload the foul sewerage system. |  |  |  |  |  |  |  |  |  |  |  |
|  | Precautions to be taken at any time of year when rainstorms are likely, actions to be taken when a rainstorm is imminent or forecast and actions to be taken during or after rainstorms. |  |  |  |  |  |  |  |  |  |  |  |
|  | Wastewater from Concrete Batching Precast Concrete Casting |  |  |  | Yes | No |  | N/A | Yes |  |  |  |
|  | Wastewater generated from the washing down of mixer trucks and drum mixers and similar equipment should wherever practicable be recycled. The discharge of wastewater should be kept to a minimum. |  |  |  |  |  |  |  |  |  |  |  |
|  | To prevent pollution from wastewater overflow, the pump sump of any water recycling system should be provided with an on-line standby pump of adequate capacity and with automatic alternating devices. |  |  |  |  |  |  |  |  |  |  |  |
|  | Under normal circumstances, surplus wastewater may be discharged into foul sewers after treatment in silt removal and pH adjustment facilities (to within the pH range of 6 to l0). Disposal of wastewater into storm drains will require more elaborate treatment. Surface run-off should be segregated from the concrete batching plant and casting yard area as much as possible， and diverted to the stormwater drainage system. Surface run-off contaminated by materials in a concrete batching plant or casting yard should be adequately treated before disposal into stormwater drains. |  |  |  |  |  |  |  |  |  |  |  |
|  | Wheel Washing Water |  |  |  | Yes | No |  | N/A | Yes |  |  |  |
|  | All vehicles and plant should be cleaned before they leave a construction site to ensure no earth, mud, debris and the like is deposited by them on roads. A wheel washing bay should be provided at every site exit if practicable and wash-water should have sand and silt settled out or removed before discharging into storm drains. The section of construction road between the wheel washing bay and the public road should be paved with backfall to reduce vehicle tracking of soil and to prevent site run-off from entering public road drains. |  |  |  |  |  |  |  |  |  |  |  |
|  | Wastewater from Building Construction |  |  |  | Yes | No |  | N/A | Yes |  |  |  |
|  | Before commencing any demolition works, all sewer and drainage connexions should be sealed to prevent building debris, soil, sand etc. from entering public sewers/drains. |  |  |  |  |  |  |  |  |  |  |  |
|  | Wastewater generated from building construction activities including concreting, plastering, internal decoration, cleaning of works and similar activities should not be discharged into the stormwater drainage system. If the wastewater is to be discharged into foul sewers, it should undergo the removal of settleable solids in a silt removal facility, and pH adjustment as necessary. |  |  |  |  |  |  |  |  |  |  |  |
|  | Acid Cleaning, Etching and Pickling Wastewater |  |  |  | Yes | No |  | N/A | Yes |  |  |  |
|  | Acidic wastewater generated from acid cleaning， etching， pickling and similar activities should be neutralized to within the pH range of 6 to l0 before discharging into foul sewers. If there is no public foul sewer in the vicinity， the neutralized wastewater should be tankered off site for disposal into foul sewers or treated to a standard acceptable to storm drains and the receiving waters. |  |  |  |  |  |  |  |  |  |  |  |