| Project Name: |  |  |  |  |  |  |  |  |  | PA submission |  | FA submission |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Project Code: |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | CIR Applied |  |  | Y |  |  | N |
|  |  |  |  |  |  |  |  | CIR Number |  |  |  |  |  |  |
| Extent of Application: |  |  |  | Whole building DC developments |  |  |  |  |  |  |  |  |  |  |
| Objectives: |  |  |  | Encourage building design to adopt measures to mitigate urban heat effect. |  |  |  |  |  |  |  |  |  |  |
| Credit Requirement: |  |  |  | For Site area <1000m2 (a) Urban Design Guidelines Chapter 11 1 credit point for implementing at least 2 site level strategies under Section 11 of Hong Kong Planning Standards and Guidelines Chapter 11 Urban Design Guidelines. For Site area ≥ 1000m2 (a) Sustainable Building Design Measures 1 credit point for providing shade on at least 5% of the site area and at least 50% of non-roof impervious surfaces on the site (parking, walkways, plazas) using light coloured high-albedo materials (albedo of at least 0.4). 1 credit point for demonstrating compliance with prescribed requirements of the SBD Guidelines as promulgated in the PNAP APP-152; and 1 additional Bonus credit point for demonstrating with relevant prescriptive requirements with enhanced performances. (b) Tree Coverage 2 to 3 Bonus credit points for demonstrating that at least 10% or 20% of the total Site Area is provided with tree coverage. (c) Air Ventilation Assessment (AVA) For conducting an AVA by wind tunnel or Computational Fluid Dynamics (CFD) according to the prevailing AVA methodology introduced by the Government demonstrating that better or equivalent ventilation performances than a baseline case: 1 credit point for demonstrating annual wind condition. 1 credit point for demonstrating summer wind condition. (d) Intra Urban Heat Island Study 2 Bonus credit points for conducting an Intra Urban Heat Island Study demonstrating that a maximum Intra-Urban Heat Index (difference between Turban and Tmet) in summer is less than 0.8 °C. |  |  |  |  |  |  |  |  |  |  |
| Credit(s) Attainable: |  |  |  |  |  | For Site area < 1,000m2: 1 ; For Site area ≥ 1,000m2: 4 + 6 Bonus |  |  |  |  |  |  |  |  |
| Credit(s) anticipated for this submission: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMPLIANCE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The Project Site area is < 1000m2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 or more site level strategies under Section 11 of Hong Kong Planning Standards and Guidelines Chapter 11 Urban Design Guidelines will be*/ are implemented, including: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The Project Site area is ≥ 1000m2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Sustainable Building Design Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Shade on at least 5% of the site area and at least 50% of non-roof impervious surfaces on the site (parking, walkways, plazas) using light coloured high-albedo materials (albedo of at least 0.4) will be*/ are provided. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Prescribed requirements of the SBD Guidelines as promulgated in the PNAP APP-152 will be*/ are complied with. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Prescribed enhanced performances of the SBD Guidelines as promulgated in the PNAP APP-152 will be*/ are complied with. |  |  |  |  |  |  |  |  |  |
|  | - Tree Coverage |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 10% or more of the total Site Area is provided with tree coverage. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 20% or more of the total Site Area is provided with tree coverage. |  |  |  |  |  |  |  |  |  |  |  |
|  | - Air Ventilation Assessment (AVA) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Better or equivalent annual wind condition will be*/ is demonstrated |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Better or equivalent annual summer condition will be*/ is demonstrated |  |  |  |  |  |  |  |  |  |  |  |
|  | - Intra Urban Heat Island Study |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Intra Urban Heat Index = Max [T(t)urban – T(t)met] |  |  |  | : |  |  |  |  |  | < 0.8 |  |

SUBMITTALS

- Sustainable Building Design Measures

| Supporting Documents |  | PA | FA |
| --- | --- | --- | --- |
| SS-03-01a_00 | BEAM Plus NDC submission template for SS-03-01 |  |  |
| For SS-03-01a with site area < 1,000 m2, please provide the following: |  |  |  |
| SS-03-01a_01 | Narrative and layout plan with markup to demonstrate the site planning comply with the strategies under Section 11 of HKPSG |  | # |
| For SS-03-01a (1) with site area ≥ 1,000 m2, please provide the following: |  |  |  |
| SS-03-01a_02 | Scaled drawings and calculations to demonstrate compliance with the relevant prescriptive requirements of the light coloured high-albedo materials on non-roof impervious surfaces. |  |  |
| SS-03-01a_03 | Material catalogues/test reports showing albedo values of the surface materials. |  |  |
| SS-03-01a_04 | A schedule of external materials, their proposed finishes treatments, and albedo requirements of the materials. |  |  |
| SS-03-01a_05 | Tender specifications on the albedo requirements of the materials |  |  |
| For SS-03-01a (2) & (3), please provide the following: |  |  |  |
| SS-03-01a_06 | Scaled drawings and calculations to demonstrate compliance with relevant prescriptive requirements of the SBD Guidelines |  |  |
| For SS-03-01a (2), please provide the following: |  |  |  |
| SS-03-01a_07 | Tender specifications on the SRI requirement of the roof materials (if applicable) |  |  |
| SS-03-01a_08 | Material catalogues/ test reports showing SRI values of the roof materials (if applicable) |  |  |
| For SS-03-01a (3), please provide the following: |  |  |  |
| SS-03-01a_09 | Landscape plans, sections, planting schedules and extracts of relevant supporting documents showing soft landscape layout, plan density, topsoil of all planted areas for trees, shrubs and grass/groundcover and demonstrating the use of drought-tolerant plant species |  |  |
| SS-03-01a_10 | Live load calculation of roof (if planting is provided on the roof) |  |  |
| SS-03-01a_11 | Maintenance plan of the greenery |  |  |
| SS-03-01a_12 | Dated photos of the as-built soft landscape works |  |  |
| # For SS-03-01a for project with site area < 1,000 m², the supporting document is not required in FA if the credit is achieved in PA |  |  |  |

- Tree Coverage

| Supporting Documents |  | PA | FA |
| --- | --- | --- | --- |
| SS-03-01b_00 | BEAM Plus NDC submission template for SS-03-01 |  |  |
| SS-03-01b_01 | Landscape plans, sections, planting schedules and extracts of relevant supporting documents showing the soft landscape layout of all planted areas for trees |  |  |
| SS-03-01b_02 | Summary for the total and breakdowns of tree coverage areas and evidence for tree diameters prediction |  |  |
| SS-03-01b_03 | Live load calculation of roof (if tree planting is provided on the roof) |  |  |
| SS-03-01b_04 | Maintenance plan of the trees |  |  |
| SS-03-01b_05 | Dated photos of the as-built soft landscape works |  |  |

- Air Ventilation Assessment (AVA)

| Supporting Documents |  | PA | FA |
| --- | --- | --- | --- |
| SS-03-01c_00 | BEAM Plus NDC submission template for SS-03-01 |  |  |
| For compliance route 1, please provide the followings: |  |  |  |
| SS-03-01c_01 | Air Ventilation Assessment Report |  | # |
| SS-03-01c_02 | Validation Report of the simulation software |  | # |
| SS-03-01c_03 | CV of the professional as per requirements in the assessment |  | # |
| For compliance route 2, please provide the followings: |  |  |  |
| SS-03-01c_04 | Wind Tunnel Test Report |  | # |
| # The supporting documents are not required in FA if the credit is achieved in PA |  |  |  |

- Intra Urban Heat Island Study

| Supporting Documents |  | PA | FA |
| --- | --- | --- | --- |
| SS-03-01d_00 | BEAM Plus New DCs submission template for SS-03-01d |  |  |
| SS-03-01d_01 | Intra-urban Heat Island Study report |  | # |
| SS-03-01d_02 | CV of the professional as per requirements in the assessment |  | # |
| SS-03-01d_03 | Landscape plans, sections planting schedules and extracts of relevant supporting documents showing soft landscape layout of all planted areas for trees |  | # |
| SS-03-01d_04 | Summary for the total and breakdowns of tree coverage areas and evidence for tree diameters prediction |  | # |
| SS-03-01d_05 | Validation Report of the simulation software |  | # |
| SS-03-01d_06 | Dated photos of the as-built soft landscape works |  |  |
| # The supporting documents are not required in FA if the Bonus credits are achieved in PA |  |  |  |

SPECIAL CIRCUMSTANCE

Describe the circumstance limiting the project team’s ability to complete this form or provide the submittals required in this template. Be sure to reference what additional documentation has been provided, if any. Non-standard documentation will be considered upon its merits (Optional)