| Credit Requirement: |  |  | a) Benchmarking For applicable types of Buildings: Credit(s) can be achieved based on the benchmarking results obtained from EMSD Benchmarking Tool. No. of Credit(s); 1; 3; 5; 1 Bonus; 2 Bonus. Percentile; 50th; 40th; 30th; 20th; 10th. Alternative for Commercial Buildings: Credit(s) can be achieved based on the label obtained from HKGBC Benchmarking & Energy Saving Tool – Office Buildings (HK BESTOF). No. of Credit(s); 1; 3; 5; 1 Bonus; 2 Bonus. HK BESTOF; Green; Bronze; Silver; Gold; Platinum. b) Self-improvement Credit(s) can be achieved based on the reduction percentage by comparing electricity bill/ metering data in the category determined in part a) Benchmarking. (Baseline year can be any year within 5 years) - For building ranked at the 40th percentile or below under EMSD Benchmarking Tool/ ”Bronze” or below label obtained from HK BESTOF: No. of Credit(s); 1; 2; 3; 4. Annual energy use reduction; 2%; 4%; 6%; 8%. - For buildings ranked at the 20th or 30th percentile under EMSD Benchmarking Tool/ ”Gold” or “Silver” label obtained from HK BESTOF: No. of Credit(s); 1; 2; 3; 4. Annual energy use reduction; 1%; 2%; 3%; 4%. - For building ranked at the 10th percentile under EMSD Benchmarking Tool or “Platinum” label obtained from HK BESTOF: No. of Credit(s); 1; 2; 3; 4. Annual energy use reduction; 0.5%; 1%; 1.5%; 2%. - For buildings which are excluded in part (a) Benchmarking: No. of Credit(s); 1; 2; 3; 4; 1 Bonus; 2 Bonus. Annual energy use reduction; 0.5%; 1%; 1.5%; 2%; 6%; 8%. c) Peak Electricity Demand Reduction Credit(s) can be achieved based on the reduction percentage in the peak electricity demand. No. of Credit(s); 1; 2; 3. Commercial and Hotel Buildings; 10%; 15%; 20%. Educational Buildings (Centralised Air-conditioning System); 10%; 15%; 20%. Educational Buildings (Unitary Air-conditioner); 5%; 10%; 15%. Other Building Types; 5%; 10%; 15%. |
| --- | --- | --- | --- |
| Credit Attainable: |  |  |  |  | 12 + 2 Bonus |  |  |
| Credit(s) anticipated for this submission: |  |  |  |  |  |  |  |
| Type of Building: |  |  |  |  |  |  |  |
| Result obtained from EMSD Benchmarking Tool / Label obtained from HK BESTOF: |  |  |  |  |  |  |  |
| Annual energy use reduction: |  |  |  |  |  | % |  |
| Peak Electricity Demand Reduction: |  |  |  |  |  | % |  |
| Explanation to fulfill BEAM Plus criteria: |  |  |  |  |  |  |  |
| Enclosures: a) Benchmarking |  |  |  |  |  |  |  |
| ☐ | Screenshots of the input for the benchmarking and relevant supporting documents. |  |  |  |  |  |  |
| ☐ | Result obtained from EMSD/ HK BESTOF. |  |  |  |  |  |  |
| ☐ | Other supporting documents, please specify. |  |  |  |  |  |  |
| b) Self-improvement |  |  |  |  |  |  |  |
| ☐ | Spreadsheet showing the energy consumption extracted from the bills and the calculation showing the percentage of reduction. |  |  |  |  |  |  |
| ☐ | Other supporting documents, please specify. |  |  |  |  |  |  |
| c) Peak Electricity Demand Reduction |  |  |  |  |  |  |  |
| ☐ | Electricity bills. |  |  |  |  |  |  |
| ☐ | Spreadsheet showing the electricity data extracted from the bills and the calculation showing the reduction percentage. |  |  |  |  |  |  |
| ☐ | Other supporting documents, please specify. |  |  |  |  |  |  |