| Credit Requirements: |  |  |  | Up to a maximum of 14 credits for using energy efficient HVAC&R and lighting systems and controls that reduce energy consumption |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Credit Attainable: |  |  |  |  | 14 |  |
| Credit(s) anticipated for this submission: |  |  |  |  |  |  |
| Enclosure |  |  |  |  |  |  |
|  | Narrative, describing the engineering installation features; |  |  |  |  |  |
|  | Drawings, engineering with “As-built” BSI floor plans, schematics, with features indicated; |  |  |  |  |  |
|  | EMSD Lighting Form (showing the percentage LPD reduction); |  |  |  |  |  |
|  | Manufacturer’s catalogue, technical data sheet (for feature/equipment installed) |  |  |  |  |  |
|  | Records with all T&C records demonstrating the relevant systems, set points, criteria; |  |  |  |  |  |
|  | Record Photographs with “as built” installations with features indicated; |  |  |  |  |  |
|  | Signature by HKSAR Registered Energy Assessor (all documents) |  |  |  |  |  |
| HVAC&R Features |  |  |  |  |  |  |
|  |  | 1 credit for appropriate zoning and thermostat distribution; |  |  |  |  |
|  |  | 1 credit for occupancy sensors and/or programmable timer controlling HVAC operation; |  |  |  |  |
|  |  | 1 credit for automatic blinds controlled by daylight sensor; |  |  |  |  |
|  |  | 1 credit for ceiling, wall mounted circulation fans; |  |  |  |  |
|  |  | 1 credit for fan coil units with variable speed drive, or high efficiency motors; |  |  |  |  |
|  |  | 1 credit for high efficiency air conditioning units; |  |  |  |  |
|  |  | 1 credit for heat recovery system; |  |  |  |  |
|  |  | 1 credit for CO2 sensors controlling fresh air rate; |  |  |  |  |
|  |  | 1 credit for complying with the installation positions for air-conditioning units; |  |  |  |  |
|  |  | 1 credit for HVACR system operating with free cooling under enthalpy control; |  |  |  |  |
|  |  | 1 credit for openable windows for natural ventilation; |  |  |  |  |
| Electrical Features |  |  |  |  |  |  |
|  |  |  | 1 credit for reducing LPD by 10% |  |  |  |
|  |  |  | 2 credit for reducing LPD by 15% |  |  |  |
|  |  |  | 3 credit for reducing LPD by 20% |  |  |  |
|  |  |  | 4 credit for reducing LPD by 25% |  |  |  |
|  |  |  | 5 credit for reducing LPD by 30% |  |  |  |
|  |  |  | 6 credit for reducing LPD by 35% |  |  |  |
|  |  |  | 1 credit for appropriate zoning and manual control; |  |  |  |
|  |  |  | 1 credit for daylight dimming controls; |  |  |  |
|  |  |  | 1 credit for occupancy sensor controls; |  |  |  |
|  |  |  | 1 credit for task light for every workstation; |  |  |  |
|  |  |  | 1 credit for entrance master switch; |  |  |  |
|  |  |  | 1 credit for compliance with voluntary lighting code; |  |  |  |

Use CIR to submit additional options for BSL consideration;