| Credit Requirements: |  | - Stormwater Management through Infiltration i. Hard landscaping 1 credit point for using pervious materials for a minimum of 50% of hard landscaped areas. AND ii. Soft landscaping 1 credit point for providing appropriate planting on site equivalent to at least 30% of the Site Area. OR 2 credit points for providing appropriate planting on site equivalent to at least 40% of the Site Area. - Stormwater Management through Temporary Storage Credit points are awarded based on the provision of temporary storage on site, which can be in the form of detention basins and / or storage tanks. The storage volume is calculated based on the site area: Required storage volume per 1000m2; No. of credit points. 30m3; 1 credit point. 60m3; 2 credit points. 90m3; 2 credit points + 1 Bonus credit point. If the site area is divided into smaller sub-catchment areas, credit points will be awarded when the temporary storage volume required for each sub-catchment exceeds the above criteria. The temporary storage volume as suggested above should be capable of detaining the runoff resulting from the following rainfall events: Provided storage volume per 1000m2; Rainfall Event. 30m3; 35 mm per hour for one hour. 60m3; 70 mm per hour for one hour. 90m3; 105 mm per hour for one hour. Hong Kong Observatory has implemented a Rainstorm Warning System which contains three levels of warning: Amber, Red and Black. The Amber signal means heavy rain has fallen or is expected to fall generally over Hong Kong, exceeding 30 millimetres in an hour, and is likely to continue. The Red signal means heavy rain has fallen or is expected to fall generally over Hong Kong, exceeding 50 millimetres in an hour, and is likely to continue. The Black signal means Very heavy rain has fallen or is expected to fall generally over Hong Kong, exceeding 70 millimetres in an hour, and is likely to continue. |
| --- | --- | --- |
| Prerequisites: |  | For residential premises with a site area larger than 1,000 m², it is required to demonstrate compliance with appropriate planting on site equivalent to at least 20% of the residential site Area. |  |
| Project Name: | Enter project name |  |
| Project Code: | Enter project code |  |

| Credit point(s) attainable: | 5 + 1B |
| --- | --- |
| Credit point(s) anticipated for this submission: | Enter anticipated points |

Please confirm:

☐    I hereby undertake that the captioned project fulfils the prerequisites of WA 2.

- Stormwater Management through Infiltration

Please check the applicable box (☐) with a cross (☒) for each item.

| Yes | No |  |  |
| --- | --- | --- | --- |
| ☐ | ☐ |  | Pervious materials are used to cover a minimum of 50% of hard landscape areas. |
| ☐ | ☐ |  | Appropriate planting covers more than 30%, but less than 40% of the Site area |
| ☐ | ☐ |  | Appropriate planting covers more than 40% of the Site area |

- Stormwater Management through Temporary Storage

| Total Site Area | : | Enter amount. m2 |
| --- | --- | --- |
| Volume of the total temporary storage provided | : | Enter amount. m3 |
| Volume of the temporary storage per 1000 m2 of the site area | : | Enter amount. m3 |
| Number of temporary storage provided on-site | : | Enter amount. |
| Types of the temporary storage provided | : | Click here to enter text. |
| Does the outlet of the temporary storage open or close during heavy rainfall? | : | Open / Close (delete the inappropriate) |
| If the outlet is open, what is the outflow rate per second per 1000 m2 of the site area? | : | Enter amount. m3/s |

Explanation to fulfil BEAM Plus criteria:

Click here to enter text.

Enclosures:

Please complete each item with a cross (☒);

- Stormwater Management through Infiltration

☐ 	A report, prepared by a suitably qualified person, on the preliminary Landscape Plan or outlined Landscape Design Strategy for the Site.

☐ 	Scale drawing(s) on an A3 size sheet with locations, areas and plan for proposed hard landscaping and soft landscaping areas within the site

☐ 	Other supporting documents, please specify.

Describe and list other supporting documents.

- Stormwater Management through Temporary Storage

☐ 	A report, prepared by a suitably qualified person, on the stormwater management through temporary storage with calculation and justifications for storage volume of rainwater on-site as well as topographical information of the site, in accordance with credit requirements of WA 2b

☐ 	Scale drawing(s) on an A3 size sheet with locations, areas and plan for proposed temporary storage within the Site

☐ 	Other supporting documents, please specify.

Describe and list other supporting documents.