| Project Name: |  |  |  | Project Code: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Log Period: (yyyy/mm) |  | to |  | Construction Stage |  | Demolition |
|  | Note: Each Log Sheet should not cover period longer than 3 months. |  |  |  |  | Foundation |
|  |  |  |  |  |  | Superstructure |

| IDCM 7a - Proactive dust control provisions to minimize air pollution during construction. |  | Properly carried out? |  |  | Rectifying Measures Required? |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Material Handling | Yes | No | N/A | Yes |  |
|  | Watering is undertaken whenever necessary for the following: |  |  |  |  |  |
|  | - Unpaved areas (e.g. unpaved main haul road) |  |  |  |  |  |
|  | - Access roads |  |  |  |  |  |
|  | - Construction areas |  |  |  |  |  |
|  | - Any dusty materials before loading and unloading |  |  |  |  |  |
|  | - Stockpile of dusty materials |  |  |  |  |  |
|  | - Debris chute |  |  |  |  |  |
|  | Water sprays are either fixed or mobile to allow individual areas to be wetted as and when required |  |  |  |  |  |
|  | Application of suitable wetting agents (e.g. dust suppression chemicals) is used in addition to water if necessary |  |  |  |  |  |
|  | Debris is covered entirely by impervious sheeting or stored in a debris collection area sheltered on the top and the 3 sides |  |  |  |  |  |
|  | Every debris chute shall be enclosed by impervious sheeting or similar materials |  |  |  |  |  |
|  | Conveyor belts are fitted within windboards |  |  |  |  |  |
|  | Conveyor transfer points and hopper discharges are enclosed |  |  |  |  |  |
|  | Drop heights are minimised from all conveyors |  |  |  |  |  |
|  | Materials that have the potential to create dust are totally enclosed |  |  |  |  |  |
|  | Storage time of materials on site is minimised |  |  |  |  |  |
|  | Bulk crushing and/or material storage takes place inside sheds |  |  |  |  |  |
|  | Skip hoist is enclosed by impervious sheeting |  |  |  |  |  |
|  | Debris chute is enclosed by impervious sheeting |  |  |  |  |  |
|  | Debris collection area is sheltered on the top and the 3 sides |  |  |  |  |  |
|  | Hoarding/ Dust Screen | Yes | No | N/A | Yes |  |
|  | Hoarding of not less than 2.4 m high from ground level is provided along the building site boundary |  |  |  |  |  |
|  | Effective dust screens, sheeting or netting is provided to enclose any scaffolding built around the perimeter of the building under construction |  |  |  |  |  |
|  | Screening of dust generating activities through the use of wind breaks |  |  |  |  |  |
|  | Excavation | Yes | No | N/A | Yes |  |
|  | Working area of any excavation or earth moving operation is sprayed with water immediately before, during and immediately after the operation |  |  |  |  |  |
|  | Exposed earth is properly treated by compaction, turfing, hydroseeding, vegetation planting or sealing with suitable surface stabiliser within 6 months after the last construction activity on the site |  |  |  |  |  |
|  | Any spoil, debris or material and silt are removed and the affected land and areas are restored to the natural state |  |  |  |  |  |
|  | Vehicle Dust/ Emission | Yes | No | N/A | Yes |  |
|  | Heights from which excavated materials are dropped during loading/unloading are minimised |  |  |  |  |  |
|  | Three-sided roofed enclosure with a flexible curtain across the entry is provided when dusty materials are discharged to vehicles from a conveying system at a fixed transfer point |  |  |  |  |  |
|  | Surface of the areas within the site where there is regular movement of vehicles are paved and any loose surface material is removed |  |  |  |  |  |
|  | Vehicles within the site are restricted to a maximum speed of 8 kph and speed limit signage is put up at appropriate locations |  |  |  |  |  |
|  | Haulage and delivery vehicles are confined to designated roadways inside the site |  |  |  |  |  |
|  | Materials transported off-site by vehicles are covered by tarpaulin, with the cover properly secured and extended over the edges of the side and tailboards |  |  |  |  |  |
|  | Dusty materials are dampened before transportation |  |  |  |  |  |
|  | Vehicle washing facility(ies) is/are provided at all site exist(s) to wash away any dusty materials from vehicle body and wheels before they leave the site |  |  |  |  |  |
|  | A hard-surfaced road between any washing facility and the public road is provided |  |  |  |  |  |
|  | Wheel washing facilities are inspected, maintained and cleaned on a regular basis |  |  |  |  |  |
|  | Vehicles are inspected regularly and well maintained to ensure that they are operating efficiently and that exhaust emissions are not causing nuisance |  |  |  |  |  |
|  | Vehicle engines are turned off when they are not in use |  |  |  |  |  |
|  | Settling silt/sediment in wheel washing bays is removed regularly |  |  |  |  |  |
|  | Any earth, mud and debris at the site exit is removed |  |  |  |  |  |
|  | Demolition Works | Yes | No | N/A | Yes |  |
|  | Any area in which demolition work takes place is sprayed with water immediately prior to, during and immediately after the demolition activities to maintain the entire surface wet |  |  |  |  |  |
|  | Impervious dust screens or sheeting is used to enclose the whole wall to a height of a least 1m higher than the highest level of the structure being demolished |  |  |  |  |  |
|  | Any dusty materials remaining after a stockpile is removed are wetted with water and cleared from road surfaces |  |  |  |  |  |
|  | Use of enclosed chutes for dropping demolition materials to ground level and the chutes are dampened regularly |  |  |  |  |  |
|  | If asbestos is found in the buildings or structure, special procedures as detailed in the Code of Practice on the Handling, Transportation and Disposal of Asbestos Waste must be followed |  |  |  |  |  |
|  | Drilling and Blasting | Yes | No | N/A | Yes |  |
|  | Spray water during breaking of rock or concrete |  |  |  |  |  |
|  | Areas within 30m from the blasting area shall be wetted with water prior to blasting |  |  |  |  |  |
|  | Blasting operations are well arranged and appropriate precautions are taken (e.g. use of blast nets, canvas covers and watering) |  |  |  |  |  |