| Project Name: |  |  |  | Project Code: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Log Period: (yyyy/mm) |  | to |  | Construction Stage |  | Demolition |
|  | Note: Each Log Sheet should not cover period longer than 3 months. |  |  |  |  | Foundation |
|  |  |  |  |  |  | Superstructure |

| IDCM 8 - Proactive waste management provisions during demolition/ construction. |  | Properly carried out? |  |  | Rectifying Measures Required? |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | General (Non Construction) | Yes | No | N/A | Yes |  |
|  | All work areas are cleaned regularly to remove general litter and refuse |  |  |  |  |  |
|  | General refuse and litter are stored in enclosed bins or compaction units separate from construction or chemical waste |  |  |  |  |  |
|  | Collect general waste and litter from site for disposal |  |  |  |  |  |
|  | No burning of refuse at any construction area |  |  |  |  |  |
|  | Separately labelled bins are provided to allow segregation of recyclable material |  |  |  |  |  |
|  | Save paper and recycle waste paper |  |  |  |  |  |
|  | All vehicles carrying waste are properly fitted with side and tail boards, and with tarpaulin covered |  |  |  |  |  |
|  | Training is provided to workers on waste management procedures |  |  |  |  |  |
|  | General (Construction) | Yes | No | N/A | Yes |  |
|  | Different types of construction waste generated from the site is segregated, stored, transported and disposed of separately |  |  |  |  |  |
|  | Sorting of waste is conducted on-site |  |  |  |  |  |
|  | All waste materials are segregated into categories covering: |  |  |  |  |  |
|  | - Inert portion of construction & demolition (C&D) material for reuse on-site |  |  |  |  |  |
|  | - Inert portion of C&D material to be used as public fill |  |  |  |  |  |
|  | - Remaining waste for landfill |  |  |  |  |  |
|  | - Chemical waste |  |  |  |  |  |
|  | - Packaging waste |  |  |  |  |  |
|  | - General waste |  |  |  |  |  |
|  | Separate compartments for inert (e.g. soil, rubble, sand, stone, etc.) and non-inert (e.g. wood, glass, plastics, steel and metals, organics, etc.) wastes are provided |  |  |  |  |  |
|  | Inert waste is used on site before disposed of at public fill reception facilities |  |  |  |  |  |
|  | Non-inert waste is sorted for re-use or recycling where possible before disposal at strategic landfills |  |  |  |  |  |
|  | Site fencing, scaffolding and timber for building work are reused wherever possible |  |  |  |  |  |
|  | Regular maintenance and cleaning of waste storage area |  |  |  |  |  |
|  | Excavated materials are properly treated (re-used/disposed of) according to specified procedures |  |  |  |  |  |
|  | General refuse stored in specified enclosed bins/compaction units, or reused if possible |  |  |  |  |  |
|  | Does the waste collector have any records of dumping? |  |  |  |  |  |
|  | A barge loading point is provided to facilitate transfer of suitable material to public dumps |  |  |  |  |  |
|  | Records of reuse/recycling/disposal of construction waste are properly kept |  |  |  |  |  |