| EFB No.: | EFB-A14a |
| --- | --- |
| EFB Name.: | Common Toilet Doors |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Common Toilets and WC Cubicles |
| Recommended Design Requirements: | Doors of toilets and WC cubicles should open outward or both ways. Locks and latches should be of larger sizes and be able to open from outside with a coin. |
| Assessment Criteria: | - At least one male and one female toilet door shall be: - Open outward; or - Open in both directions; or - Designed as sliding doors. - For the same toilets in (1), at least one WC cubicle door shall be: - Open outward; or - Open in both directions; or - Designed as sliding doors. - All WC cubicles shall be equipped with larger locks and latches that are easy to operate without the need of strong finger force. - All WC cubicle locks and latches shall be designed to allow easy opening from the outside using a coin or other standard item, without the need for specialised tools. - For project where only unisex toilets are provided: - At least one unisex toilet shall comply with the requirement (1); and - Requirement (2), (3) & (4) are exempted for assessment. |
| Submittals: | - EFB-A14a_01 Provide drawing(s) and/ or sample photo(s) and/ or catalogue(s) to demonstrate: - All doors of toilets are opened outward/ both ways/ sliding doors. |
|  | - EFB-A14a_02 Provide drawing(s) and/ or sample photo(s)1 and/ or catalogue(s) to demonstrate: - All doors of WC cubicles are opened outward/ both ways/ sliding doors; - All WC cubicles are equipped with larger locks and latches; - All WC cubicle locks and latches are easily opened from the outside. |
| Reference: |  |

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.