| EFB No.: | EFB-A14c |
| --- | --- |
| EFB Name.: | Emergency Alarm at Common Toilets |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Common Toilets and WC Cubicles |
| Recommended Design Requirements: | Emergency alarm system should be provided. Emergency call bell should comply with Division 17. Should pull-cord be installed, they must be extended to floor-level. |
| Assessment Criteria: | - All common toilets shall comply with the requirement. - At least one emergency alarm system shall be provided in a commonly accessible area with the toilet, allowing activation without the need to enter a cubicle. - Emergency call bells, if provided, shall emit both audible and visible alarm signals, ensuring they are easily noticeable for summoning assistance. - Emergency pull-cord, if provided, shall be extended to floor-level. - For projects where all toilets are individual standalone rooms without cubicles, providing an emergency alarm system within each toilet is regarded as meeting the requirement. |
| Submittals: | - EFB-A14c_01 Provide drawing(s) and/ or sample photo(s) to demonstrate: - Emergency alarm system is provided in a commonly accessible area within the toilet. |
|  | - EFB-A14c_02 Provide drawing(s) and/ or catalogue(s) and/ or sample video(s)2 to demonstrate: - Emergency call bells are equipped with both audible and visible alarm signals. |
|  | - EFB-A14c_03 Provide drawing(s) and/ or sample photo(s)1 to demonstrate: - Emergency pull-cord is extended to floor-level. |

Design Manual: Barrier Free Access 2008, Buildings Department, HKSAR.

At least one set of photos/ videos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.