| EFB No.: | EFB-A17a |
| --- | --- |
| EFB Name.: | Lighting Level at Common Areas |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Common Areas and Facilities |
| Recommended Design Requirements: | All common areas of a building should have an illumination level of not less than 120 lux measured at the finished floor level, and uniformity of illumination level should be maintained for any space. |
| Assessment Criteria: | - All common areas of a building shall have an illumination level of not less than 120 lux, measured at the finished floor level. - Uniformity of illumination levels shall be maintained with no visible uneven lighting throughout the common areas. |
| Submittals: | - EFB-A17a_01 Provide at least one simulation result and/ or on-site measurement result for each area type to demonstrate: - The illumination at finished floor level in all common areas shall be at least 120 lux. |
|  | - EFB-A17a_02 Provide any one of the supporting information: - At least one photo for each area type to demonstrate there is no visible bright/ dark spot in the common areas; - At least one simulation result for each area type to demonstrate the uniformity is at least 0.4. |

Common areas include entrance lobbies, lift lobbies, corridors, and toilets.