| EFB No.: | EFB-A1a |
| --- | --- |
| EFB Name.: | Residential Bathroom Spaces |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Residential Units’ Bathrooms |
| Recommended Design Requirements: | Adequate clear space of not less than 1200 mm diameter should be allowed for turning of wheelchair and transfer of users. |
| Assessment Criteria: | - At least 20% of the residential units shall comply with the requirement. - A clear turning space with a minimum diameter of at least 1200 mm shall be provided in the bathroom(s). - Overlapping the turning space with the shower area is acceptable. - The turning space may overlap with the area beneath the wash basin, provided that the following conditions are met: - Movable base cabinets are installed beneath the wash basin; and - A clear height of 680 mm, measured from the finished floor level, is maintained to ensure unobstructed wheelchair turning. - If more than 1 bathroom is provided in the unit, at least 1 of the bathrooms shall fulfil the requirement. |
| Submittals: | - EFB-A1a_01 Complete the following table: Total no. of residential units in the project:. 20% of residential units in the project (round up to the nearest whole number):. No. of residential units that comply with the requirements:. |
|  | - EFB-A1a_02 Provide any one of the supporting information showing the total no. of residential units in the project: - Latest GBP; - Sales Brochure; - Declaration signed by a representative of the Applicant (refer to Appendix A). |
|  | - EFB-A1a_03 Provide drawing(s) and/ or sample photo(s) to demonstrate: - A minimum 1200 mm diameter turning space. |
|  | - EFB-A1a_04 Provide drawing(s) and/ or sample photo(s)2 to demonstrate: - Movable base cabinets are provided; and - A clear height of 680 mm beneath the wash basin, measured from the finished floor level, is maintained. |
| Reference: | Reference: Curating Homes: A Guide to Residential Design for Ageing in Place, CUHK Jockey Club Institute of Ageing Reference: Design Manual: Barrier Free Access 2008 (2025 Edition), Buildings Department, HKSAR |

Representative of the Applicant is defined as: (i) Building Owner; (ii) Applicant of this certification; (iii) The project Authorized Person/ BEAM Pro/ Architect; (iv) Representative from the facility management company.

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.