| EFB No.: | EFB-A1b |
| --- | --- |
| EFB Name.: | Residential Bathroom Grab Bars |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Residential Units’ Bathrooms |
| Recommended Design Requirements: | Grab bars should be provided. |
| Assessment Criteria: | - At least 20% of the residential units shall comply with the requirement. - At least one grab bar (or folding grab bar) shall be provided in the bathroom, excluding those located at bathtub or within the shower compartment. - The surface of the grab bars shall be slip-resistant, e.g. stainless steel with hair-line finish. - The grab bar(s) shall be installed at a height between 725 mm to 750 mm measured from the bottom of the bar to the finished floor level. - If more than 1 bathroom is provided in the unit, at least 1 of the bathrooms shall fulfil the requirement. |
| Submittals: | - EFB-A1b_01 Complete the following table: Total no. of residential units in the project:. 20% of residential units in the project (round up to the nearest whole number):. No. of residential units that comply with the requirements:. |
|  | - EFB-A1b_02 Provide any one of the supporting information showing the total no. of residential units in the project: - Latest GBP; - Sales Brochure; - Declaration signed by a representative of the Applicant (refer to Appendix A). |
|  | - EFB-A1b_03 Provide drawing(s) and/ or sample photo(s) and/ or catalogue(s)to demonstrate: - The grab bars with a slip-resistant design. |
|  | - EFB-A1b_04 Provide drawing(s) and/ or sample photo(s)2 to demonstrate: - The grab bar is installed at a height between 725 mm to 750 mm measured from the bottom of the bar to the finished floor level. |

Representative of the Applicant is defined as: (i) Building Owner; (ii) Applicant of this certification; (iii) The project Authorized Person/ BEAM Pro/ Architect; (iv) Representative from the facility management company.

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.