| EFB No.: | EFB-A3a |
| --- | --- |
| EFB Name.: | Residential Bathtub Seating |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Residential Units’ Bathrooms |
| Recommended Design Requirements: | Bathtub should have either built-in seat at the head end or attachable portable seat that fastens securely to the tub when needed. |
| Assessment Criteria: | - At least 20% of the residential units shall comply with the requirement. - Built-in/ attachable portable seat shall be provided in the bathtub. - The seat shall have a minimum depth of 250 mm. - The bathtub shall have a maximum height of 380 mm. - If more than 1 bathroom is provided in the unit, at least 1 of the bathrooms shall fulfil the requirement. |
| Submittals: | - EFB-A3a_01 Complete the following table: Total no. of residential units in the project:. 20% of residential units in the project (round up to the nearest whole number):. No. of residential units comply with the requirement:. |
|  | - EFB-A3a_02 Provide any one of the supporting information showing the total no. of residential units in the project: - Latest GBP; - Sales Brochure; - Declaration signed by a representative of the Applicant (refer to Appendix A). |
|  | - EFB-A3a_03 Provide drawing(s) and/ or sample photo(s) to demonstrate: - The height of the bathtub is not higher than 380 mm; - The built-in/ attachable portable seat have a depth of at least 250 mm. |
| Reference: | Reference: Design Manual: Barrier Free Access 2008 (2025 Edition), Buildings Department, HKSAR |

Representative of the Applicant is defined as: (i) Building Owner; (ii) Applicant of this certification; (iii) The project Authorized Person/ BEAM Pro/ Architect; (iv) Representative from the facility management company

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.