| EFB No.: | EFB-A3b |
| --- | --- |
| EFB Name.: | Residential Shower Seating |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Residential Units’ Bathrooms |
| Recommended Design Requirements: | Showers should be of size not less than 1500 mm x 900 mm with a folding seat to allow transfer of users. |
| Assessment Criteria: | - At least 20% of the residential units shall comply with the requirement. - Folding seat shall be provided in the shower(s). - The seat shall not be less than 550 mm in width and 450 mm in depth. - The seat shall be installed at a height between 430 mm and 480 mm from the top of the seat to the finished floor level. - If more than 1 bathroom is provided in the unit, at least 1 of the bathrooms shall fulfil the requirement. |
| Submittals: | - EFB-A3b_01 Complete the following table: Total no. of residential units in the project:. 20% of residential units in the project (round up to the nearest whole number):. No. of residential units that comply with the requirements:. |
|  | - EFB-A3b_02 Provide any one of the supporting information showing the total no. of residential units in the project: - Latest GBP; - Sales Brochure; - Declaration signed by a representative of the Applicant (refer to Appendix A). |
|  | - EFB-A3b_03 Provide drawing(s) and/ or catalogue(s) and/ or sample photo(s) to demonstrate: - The folding seat is at least 550 mm x 450 mm; - The height of the folding seat is installed at between 430 mm and 480 mm above the finished floor level. |
| Reference: | Reference: Design Manual: Barrier Free Access 2008 (2025 Edition), Buildings Department, HKSAR |

Representative of the Applicant is defined as: (i) Building Owner; (ii) Applicant of this certification; (iii) The project Authorized Person/ BEAM Pro/ Architect; (iv) Representative from the facility management company

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.