| EFB No.: | EFB-A3c |
| --- | --- |
| EFB Name.: | Residential Bathtub and Shower Grab Bars |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Residential Units’ Bathrooms |
| Recommended Design Requirements: | Grab bars should be provided. |
| Assessment Criteria: | - At least 20% of the residential units shall comply with the requirement. - Grab bar(s) shall be provided in the bathtub(s)/ shower(s). - The surface of the grab bars shall be slip-resistant, e.g. stainless steel with hair-line finish. - For bathtubs, the following shall be provided: - One horizontal grab bar; and - One vertical grab bar. - For shower compartments, two additional vertical grab bars shall be provided. The grab bars shall meet the following requirement: - A minimum length of 750 mm; - Installed at a height between 700 mm and 800 mm measured from the bottom of the bars to the finished floor level; and - Installed adjacent to the shower seat such that one is mounted beside the shower seat and another one is mounted not more than 450 mm from the edge of the shower seat. - If more than 1 bathroom is provided in the unit, at least 1 of the bathrooms shall fulfil the requirement. |
| Submittals: | - EFB-A3c_01 Complete the following table: Total no. of residential units in the project:. 20% of residential units in the project (round up to the nearest whole number):. No. of residential units that comply with the requirements:. |
|  | - EFB-A3c_02 Provide any one of the supporting information showing the total no. of residential units in the project: - Latest GBP; - Sales Brochure; - Declaration signed by a representative of the Applicant (refer to Appendix A). |
|  | - EFB-A3c_03 Provide drawing(s) and/ or sample photo(s) and/ or catalogue(s)to demonstrate: - The grab bars with a slip-resistant design. |
|  | - EFB-A3c_04 If grab bars are provided in bathtubs, provide drawing(s) and/ or sample photo(s)2 to demonstrate: - At least one horizontal and one vertical grab bars are provided. |
|  | - EFB-A3c_05 If grab bars are provided in shower compartments, provide drawing(s) and/ or sample photo(s)2 to demonstrate: - Two additional vertical grab bars with a minimum length of 750 mm are provided; - The vertical grab bars are installed at a height between 700 mm and 800 mm, measured from the bottom of the bars to the finished floor level; - Positioned adjacent to the shower seat, with one grab bar mounted beside the shower seat and the other located no more than 450 mm from the edge of the shower seat. |
| Reference: | Reference: Design Manual: Barrier Free Access 2008 (2025 Edition), Buildings Department, HKSAR |

Representative of the Applicant is defined as: (i) Building Owner; (ii) Applicant of this certification; (iii) The project Authorized Person/ BEAM Pro/ Architect; (iv) Representative from the facility management company

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.