| EFB No.: | EFB-A4a |
| --- | --- |
| EFB Name.: | Residential Bathroom Floor Surfaces |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Residential Units’ Bathrooms |
| Recommended Design Requirements: | Floor surfaces should comply with Division 4. Slip-resistant floor finishes should be used. Shiny and reflective floors such as marble, glazed tiles and the like should be avoided. |
| Assessment Criteria: | - At least 20% of the residential units shall comply with the requirement. - Slip-resistant floor finishes shall be provided in the bathrooms. - The floor surfaces shall meet the following minimum slip-resistance requirements: - A static coefficient of friction (SCOF) equal to or greater than 0.5; or - A ramp test rating (DIN 51130) of at least R10. - If more than 1 bathroom is provided in the unit, at least 1 of the bathrooms shall fulfil the requirement. |
| Submittals: | - EFB-A4a_01 Complete the following table: Total no. of residential units in the project:. 20% of residential units in the project (round up to the nearest whole number):. No. of residential units that comply with the requirements:. |
|  | - EFB-A4a_02 Provide any one of the supporting information showing the total no. of residential units in the project: - Latest GBP; - Sales Brochure; - Declaration signed by a representative of the Applicant (refer to Appendix A). |
|  | - EFB-A4a_03 Provide catalogue(s) and/ or laboratory report(s) to demonstrate: - The SCOF/ ramp test rating of the floor finishes is equal to or greater than 0.5/ R10. |

Buildings Department’s Design Manual Barrier Free Access 2008

Representative of the Applicant is defined as: (i) Building Owner; (ii) Applicant of this certification; (iii) The project Authorized Person/ BEAM Pro/ Architect; (iv) Representative from the facility management company