| EFB No.: | EFB-A5a |
| --- | --- |
| EFB Name.: | Residential Light Switches |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Residential Units |
| Recommended Design Requirements: | Double-switching systems should be considered. Switches and controls should be provided at reachable level. For bedrooms, switches and controls should be provided near bedside to avoid crossing the room in the dark. |
| Assessment Criteria: | - At least 20% of the residential units shall comply with the requirement. - At least one set of switch(s) for each room shall comply with the requirement. - Light switch(s) system shall either be: - Double-switching systems, with one switch positioned at standing height and the other at sitting height; or - Positioned between 1000 mm to 1300 mm above the finished floor level. - For bedrooms: - At least one switch shall be positioned near to the notional location of the bed; or - A timer-controlled lighting switch should be provided to prevent elderly individuals from needing to cross the room in the dark to turn off the light. - If more than 1 bedroom is provided in the unit, at least 1 of the bedrooms shall fulfil the requirement. |
| Submittals: | - EFB-A5a_01 Complete the following table: Total no. of residential units in the project:. 20% of residential units in the project (round up to the nearest whole number):. No. of residential units that comply with the requirements:. |

|  | - EFB-A5a_02 Provide any one of the supporting information showing the total no. of residential units in the project: - Latest GBP; - Sales Brochure; - Declaration signed by a representative of the Applicant (refer to Appendix A). |
| --- | --- |
|  | - EFB-A5a_03 Provide drawing(s) and/ or sample photo(s) to demonstrate: - Double-switching systems are provided or the switch(s) are positioned at a height between 1000 mm to 1300 mm above floor level. |
|  | - EFB-A5a_04 Provide any one of the supporting information: - Drawing(s) and/ or sample photo(s)2 to demonstrate at least one switch are positioned near to the notional location of the bed; or - Drawing(s) and/ or catalogue(s) to demonstrate a timer-controlled switch is provided. |

Representative of the Applicant is defined as: (i) Building Owner; (ii) Applicant of this certification; (iii) The project Authorized Person/ BEAM Pro/ Architect; (iv) Representative from the facility management company

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.