| EFB No.: | EFB-A5c |
| --- | --- |
| EFB Name.: | Residential Overhead Storage |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Residential Units |
| Recommended Design Requirements: | Cupboards should not be installed at high levels. Overhead cabinets should be avoided. Pull-down shelves should be provided for cupboards at high level or overhead cabinets. |
| Assessment Criteria: | - At least 20% of the residential units shall comply with the requirement. - No overhead storage cabinet shall be provided in the kitchen(s) and bathroom(s). - If overhead storage cabinet(s) are provided, drop-down or pull-down shelf mechanism shall be provided. - If more than 1 bathroom/ kitchen is provided in the unit, at least 1 of the bathrooms/ kitchens shall fulfil the requirement. |
| Submittals: | - EFB-A5c_01 Complete the following table: Total no. of residential units in the project:. 20% of residential units in the project (round up to the nearest whole number):. No. of residential units that comply with the requirements:. |
|  | - EFB-A5c_02 Provide any one of the supporting information showing the total no. of residential units in the project: - Latest GBP; - Sales Brochure; - Declaration signed by a representative of the Applicant (refer to Appendix A). |
|  | - EFB-A5c_03 Provide drawing(s) and/ or sample photo(s) and/ or catalogue(s) to demonstrate: - No overhead storage cabinet are provided in the kitchen(s) and bathroom(s); or - Drop-down or pull-down shelf mechanism are provided for overhead storage cabinet. |

Overhead storage cabinet refers to a cabinet positioned at a height of 1500 mm or more above the finished floor level, measured from the bottom of the cabinet.

Representative of the Applicant is defined as: (i) Building Owner; (ii) Applicant of this certification; (iii) The project Authorized Person/ BEAM Pro/ Architect; (iv) Representative from the facility management company

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.