| EFB No.: | EFB-A6a |
| --- | --- |
| EFB Name.: | Level Difference at Commonly Accessible Routes |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Accessible Routes at Common Area |
| Recommended Design Requirements: | Accessible routes should be without steps, thresholds, small ramps or kerbs, wherever possible. Where changes in level are unavoidable, handrails or grab bars should be provided. The floor and wall surface along the level of difference should be in luminous contrast. |
| Assessment Criteria: | - The major access route(s) shall be free of steps, thresholds, small ramps, or kerbs as far as possible. - For all level differences along the major access route(s), the floor and wall surface shall be in luminous contrast (e.g. visible light/ dark differences). - When identifying the major access route(s), the circulation paths accessing to all of the following space types from the main building entrance(s) shall be considered: - Main lobby/ foyer; - Lift lobbies; - Clubhouse entrance; - Shops, restaurants, cinemas, etc.; - Toilets; - External recreational facilities. - For each podium and block/tower (as applicable), accessible routes shall be provided from the main entrance to each of the required facility types listed in Criterion 3. These routes do not need to form a single continuous path; multiple distinct accessible routes may be used. However, there shall be at least one compliant accessible route to each listed facility type, ensuring that every required destination is reachable via a route that meets all relevant accessibility requirements. This is to provide safe and inclusive access, particularly for elderly individuals. - Common corridors on typical floors are generally not included in the assessment but may be considered for inclusion at the discretion of the applicant. |
| Submittals: | - EFB-A6a_01 Provide annotated layout drawing(s) to identify: - The major access route(s); and - The location(s) where there is a level differences (e.g. steps, thresholds, small ramps, or kerbs, etc.). |
|  | - EFB-A6a_02 Provide drawing(s) and/ or photo(s) to illustrate that the floor and wall surfaces at level differences have luminous contrast. |
| Reference: | Reference: Elderly-friendly Design Guidelines, Architectural Services Department, HKSAR |

Major access route is referred to the primary pathway or circulation route that enables safe, convenient, and unobstructed movement within the building that connects key areas such as entrances, elevators, staircases, communal spaces, and essential facilities, etc. For detailed step-by-step guidance on determining the major access route, refer to Appendix B1.

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.