| EFB No.: | EFB-A6c |
| --- | --- |
| EFB Name.: | Common Staircases Treads and Risers |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Accessible Routes at Common Area |
| Recommended Design Requirements: | Steps and staircases should be designed with wider treads and lower risers. |
| Assessment Criteria: | - Steps and staircases along major access route(s) shall be: - Designed with uniform riser height and tread depth; - Riser Height shall not be greater than 150 mm; - Tread Depth shall not be less than 300 mm. - When identifying the major access route(s), the circulation paths accessing to all of the following space types from the main building entrance(s) shall be considered: - Main lobby/ foyer; - Lift lobbies; - Clubhouse entrance; - Shops, restaurants, cinemas, etc.; - Toilets; - External recreational facilities. - For each podium and block/tower (as applicable), accessible routes shall be provided from the main entrance to each of the required facility types listed in Criterion 3. These routes do not need to form a single continuous path; multiple distinct accessible routes may be used. However, there shall be at least one compliant accessible route to each listed facility type, ensuring that every required destination is reachable via a route that meets all relevant accessibility requirements. This is to provide safe and inclusive access, particularly for elderly individuals. - Back of house staircases are excluded in the assessment. |
| Submittals: | - EFB-A6c_01 Provide annotated layout drawing(s) to identify: - The major access route(s); and - The location(s) where steps/ staircases are provided. |
|  | - EFB-A6c_02 Provide drawing(s) and/ or sample photo(s) to demonstrate: - Uniform riser height and tread depth are provided; - Riser Height is less than 150 mm; - Tread Depth is greater than 300 mm. |

Major access route is referred to the primary pathway or circulation route that enables safe, convenient, and unobstructed movement within the building that connects key areas such as entrances, elevators, staircases, communal spaces, and essential facilities, etc. For detailed step-by-step guidance on determining the major access route, refer to Appendix B1.

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.