| EFB No.: | EFB-A7a |
| --- | --- |
| EFB Name.: | Handrail Design at Commonly Accessible Routes |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Accessible Routes at Common Area |
| Recommended Design Requirements: | Handrails to corridors, steps and staircases should be lowered to a height between 810 mm and 900 mm from the finished floor level to the top of the handrails. Handrails should be of materials such as timber or with plastic coated surfaces for easier grip. |
| Assessment Criteria: | - Handrails along major access route(s) shall be positioned at a height between 810 mm and 900 mm measured from the finished floor level to the top of the handrails. - Handrails shall incorporate features such as plastic-coated surfaces, textured finishes, or made of timber to enhance grip. - When identifying the major access route(s), the circulation paths accessing to all of the following space types from the main building entrance(s) shall be considered: - Main lobby/ foyer; - Lift lobbies; - Clubhouse entrance; - Shops, restaurants, cinemas, etc.; - Toilets; - External recreational facilities. - For each podium and block/tower (as applicable), accessible routes shall be provided from the main entrance to each of the required facility types listed in Criterion 3. These routes do not need to form a single continuous path; multiple distinct accessible routes may be used. However, there shall be at least one compliant accessible route to each listed facility type, ensuring that every required destination is reachable via a route that meets all relevant accessibility requirements. This is to provide safe and inclusive access, particularly for elderly individuals. - Back of house staircases are excluded in the assessment. |
| Submittals: | - EFB-A7a_01 Provide annotated layout drawing(s) to identify: - The major access route(s); and - The location(s) where handrails are provided. |
|  | - EFB-A7a_02 Provide drawing(s) and/ or sample photo(s) to demonstrate: - The height of the handrails is between 810 mm and 900 mm measured from the finished floor level to the top of the handrails. |
|  | - EFB-A7a_03 Provide catalogue(s) and/ or sample photo(s)2 to demonstrate: - The handrails design to enhance grip (e.g. plastic-coated surfaces, textured finishes, or made of timber). |

Major access route is referred to the primary pathway or circulation route that enables safe, convenient, and unobstructed movement within the building that connects key areas such as entrances, elevators, staircases, communal spaces, and essential facilities, etc. For detailed step-by-step guidance on determining the major access route, refer to Appendix B1.

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.