| EFB No.: | EFB-A8b |
| --- | --- |
| EFB Name.: | Self-Closing Doors at Commonly Accessible Routes |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Accessible Routes at Common Area |
| Recommended Design Requirements: | If door closing devices are installed, they should be designed to allow external and internal doors to be opened with horizontal forces of not more than 28N and 18N respectively. |
| Assessment Criteria: | - If door closing devices are installed along major access route(s), it shall be designed to be opened with horizontal forces of not more than 28N and 18N for external and internal doors respectively. - The door closing devices shall be provided with an adjustable closing mechanism. - When identifying the major access route(s), the circulation paths accessing to all of the following space types from the main building entrance(s) shall be considered: - Main lobby/ foyer; - Lift lobbies; - Clubhouse entrance; - Shops, restaurants, cinemas, etc.; - Toilets; - External recreational facilities. - For each podium and block/tower (as applicable), accessible routes shall be provided from the main entrance to each of the required facility types listed in Criterion 3. These routes do not need to form a single continuous path; multiple distinct accessible routes may be used. However, there shall be at least one compliant accessible route to each listed facility type, ensuring that every required destination is reachable via a route that meets all relevant accessibility requirements. This is to provide safe and inclusive access, particularly for elderly individuals. - Fire exit doors can be exempted in the assessment. |
| Submittals: | - EFB-A8b_01 Provide annotated layout drawing(s) to identify: - The major access route(s); and - The location(s) where self-closing doors are provided. |
|  | - EFB-A8b_02 Provide drawing(s) and/ or catalogue(s) to demonstrate: - Adjustable closing mechanism is provided. |
|  | - EFB-A8b_03 - Declaration signed by a representative of the Applicant (refer to Appendix A). |

Major access route is referred to the primary pathway or circulation route that enables safe, convenient, and unobstructed movement within the building that connects key areas such as entrances, elevators, staircases, communal spaces, and essential facilities, etc. For detailed step-by-step guidance on determining the major access route, refer to Appendix B1.

A self-closing door with an adjustable closing mechanism features a built-in device that automatically closes the door after it has been opened. The adjustable mechanism allows users to control the speed and force of the door's closing action, ensuring smooth and safe operation for various needs and environments.

Representative of the Applicant is defined as: (i) Building Owner; (ii) Applicant of this certification; (iii) Representative from the facility management company