| EFB No.: | EFB-A8c |
| --- | --- |
| EFB Name.: | Door Handles at Commonly Accessible Routes |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Accessible Routes at Common Area |
| Recommended Design Requirements: | Lever-type handles should be used. Knob handles should be avoided. A sample is provided below. Reference: Design Manual: Barrier Free Access 2008, Buildings Department, HKSAR |
| Assessment Criteria: | - Lever-type or push/ pull plate handles shall be provided for manually operated doors along major access route(s). - When identifying the major access route(s), the circulation paths accessing to all of the following space types from the main building entrance(s) shall be considered: - Main lobby/ foyer; - Lift lobbies; - Clubhouse entrance; - Shops, restaurants, cinemas, etc.; - Toilets; - External recreational facilities. - For each podium and block/tower (as applicable), accessible routes shall be provided from the main entrance to each of the required facility types listed in Criterion 3. These routes do not need to form a single continuous path; multiple distinct accessible routes may be used. However, there shall be at least one compliant accessible route to each listed facility type, ensuring that every required destination is reachable via a route that meets all relevant accessibility requirements. This is to provide safe and inclusive access, particularly for elderly individuals. - Fire exit doors can be exempted in the assessment. |
| Submittals: | - EFB-A8c_01 Provide annotated layout drawing(s) to identify: - The major access route(s); and - The location(s) where manually operated doors are provided. |
|  | - EFB-A8c_02 Provide drawing(s) and/ or catalogue(s) and/ or sample photo(s) to demonstrate: - Lever-type or push/ pull plate handles are provided. |

Major access route is referred to the primary pathway or circulation route that enables safe, convenient, and unobstructed movement within the building that connects key areas such as entrances, elevators, staircases, communal spaces, and essential facilities, etc. For detailed step-by-step guidance on determining the major access route, refer to Appendix B1.

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.