| EFB No.: | EFB-A9a |
| --- | --- |
| EFB Name.: | Floor Surfaces at Commonly Accessible Routes |
| Category: | A. Mobility and Accessibility |
| Extent of Application: | Accessible Routes at Common Area |
| Recommended Design Requirements: | Floor surfaces should comply with Division 4. Slip-resistant floor finishes should be used, shiny and reflective floors such as marble, glazed tiles and the like should be avoided. Open jointed pavers or aeration paver blocks with uneven or very rough surface should be avoided at external open spaces. |
| Assessment Criteria: | - The floor finishes along major access route(s) shall meet the following minimum slip-resistance requirements: - A static coefficient of friction (SCOF) greater than 0.5; or - A ramp test rating (DIN 51130) of at least R10. - When identifying the major access route(s), the circulation paths accessing to all of the following space types from the main building entrance(s) shall be considered: - Main lobby/ foyer; - Lift lobbies; - Clubhouse entrance; - Shops, restaurants, cinemas, etc.; - Toilets; - External recreational facilities. - For each podium and block/tower (as applicable), accessible routes shall be provided from the main entrance to each of the required facility types listed in Criterion 3. These routes do not need to form a single continuous path; multiple distinct accessible routes may be used. However, there shall be at least one compliant accessible route to each listed facility type, ensuring that every required destination is reachable via a route that meets all relevant accessibility requirements. This is to provide safe and inclusive access, particularly for elderly individuals. - External open spaces along major access route(s) shall not include open jointed pavers or aeration paver blocks with uneven or excessively rough surfaces. |
| Submittals: | - EFB-A9a_01 Provide annotated layout drawing(s) to identify: - The major access route(s). |
|  | - EFB-A9a_02 Provide catalogue(s) and/ or laboratory report(s) to demonstrate: - The SCOF/ ramp test rating of the floor finishes is greater than 0.5/ R10. |
|  | - EFB-A9a_03 Provide catalogue(s) and/ or sample photo(s) to demonstrate: - The floor finishes at external open spaces are without jointed pavers or aeration paver blocks with uneven or excessively rough surfaces. |

Design Manual: Barrier Free Access 2008, Buildings Department, HKSAR

Major access route is referred to the primary pathway or circulation route that enables safe, convenient, and unobstructed movement within the building that connects key areas such as entrances, elevators, staircases, communal spaces, and essential facilities, etc. For detailed step-by-step guidance on determining the major access route, refer to Appendix B1.

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.