| EFB No.: | EFB-B1a |
| --- | --- |
| EFB Name.: | Residential Unit Adaptability |
| Category: | B. Design Adaptability |
| Extent of Application: | Residential Units |
| Recommended Design Requirements: | To suit the needs of the elderly at different ages, the residential units should be convertible without the need for making structural alterations so as to accommodate the following elderly-friendly designs: To reduce possible safety risks - Raised deck or similar to overcome level difference between indoor living area and the balcony; - Handrail / grab bars in toilet / shower area; - Shower seat; and - Low threshold and curbless walk-in shower. To cater for the elderly on wheelchair - Lower door viewer for main entrance; - Wider corridor within units with a clear width of not less than 950 mm; - Large and / or two-way switches and controls at reachable level; - Maneuvering area of not less than 1200 mm diameter in main entrance, kitchen, toilet and bedroom;. - Wider internal door with a clear width of not less than 850 mm; and - Knee space under kitchen sink and wash basin. To improve daily convenience - Counter–top in kitchen with a depth of not more than 600 mm; - Adequate drainage provisions to enable adaptive design; and A sample of lower eye viewer at main entrance door and layout of elderly-friendly adaptive design is provided below. Reference: Design Manual: Barrier Free Access 2008 (2025 Edition), Buildings Department, HKSAR |
| Assessment Criteria: | - At least 20% of the residential units shall comply with the requirement. - Corridors within the unit shall have a clear width of not less than 950 mm. - A maneuvering area with a diameter of not less than 1200 mm shall be provided in the main entrance, kitchen, toilet, and bedroom. - Internal doors shall have a clear width of not less than 850 mm. - If more than 1 kitchen/ bathroom is provided in the unit, at least 1 of the kitchens/ bathrooms shall fulfil the requirement. |
| Submittals: | - EFB-B1a_01 Complete the following table: Total no. of residential units in the project:. 20% of residential units in the project (round up to the nearest whole number):. No. of residential units that comply with the requirements:. |

|  | - EFB-B1a_02 Provide any one of the supporting information showing the total no. of residential units in the project: - Latest GBP; - Sales Brochure; - Declaration signed by a representative of the Applicant (refer to Appendix A). |
| --- | --- |
|  | - EFB-B1a_03 Provide drawing(s) and/ or sample photo(s) to demonstrate: - Corridors with a minimum clear width of 950 mm; - A turning space with a minimum diameter of 1200 mm in the main entrance, kitchen, toilet, and bedroom; - Internal doorways with a minimum clear width of 850 mm. |

Representative of the Applicant is defined as: (i) Building Owner; (ii) Applicant of this certification; (iii) The project Authorized Person/ BEAM Pro/ Architect; (iv) Representative from the facility management company

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.