| EFB No.: | EFB-C1a |
| --- | --- |
| EFB Name.: | Residential Units Windows |
| Category: | C. Well-being |
| Extent of Application: | Residential Units |
| Recommended Design Requirements: | Larger windows with lower window cills should be provided to increase the amount of natural light and allow views of the outside for the elderly when seated or on wheelchair. This can have positive impact on moods and reduce feelings of depression and anxiety. Nevertheless, requirements of protective barrier should be complied with. Other concerns such as privacy and fear of height should also be considered. Sliding windows should be adopted to enhance daily use by the elderly. |
| Assessment Criteria: | - At least 20% of the residential units shall comply with the requirement. - Windows shall be lowered and positioned with a maximum height measured from the finished floor level as follows: - Bedroom: 600 mm; - Living Room: 730 mm. - Sliding windows shall be adopted to enhance ease of use for the elderly in their daily activities. - If more than 1 bedroom/ living room is provided in the unit, at least 1 of the bedrooms/ living rooms shall fulfil the requirement. - If more than 1 window is provided within the bedroom/ living room, at least 1 of the windows in each room shall fulfil the requirement. - Sliding door at balcony or utility platform accessible from the bedroom or living room shall be deemed to comply with requirement (2.1), (2.2) and (3) for the respective room. |
| Submittals: | - EFB-C1a_01 Complete the following table: Total no. of residential units in the project:. 20% of residential units in the project (round up to the nearest whole number):. No. of residential units comply with the requirement:. |
|  | - EFB-C1a_02 Provide any one of the supporting information showing the total no. of residential units in the project: - Latest GBP; - Sales Brochure; - Declaration signed by a representative of the Applicant (refer to Appendix A). |
|  | - EFB-C1a_03 Provide drawing(s) and/ or sample photo(s) to demonstrate: - The bedroom window is positioned at a maximum height of 600 mm above the finished floor level; - The living room window is positioned at a maximum height of 730 mm above the finished floor level; - Sliding windows are provided. |
| Reference: | Reference: Design Manual: Barrier Free Access 2008 (2025 Edition), Buildings Department, HKSAR |

Representative of the Applicant is defined as: (i) Building Owner; (ii) Applicant of this certification; (iii) The project Authorized Person/ BEAM Pro/ Architect; (iv) Representative from the facility management company

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.