| EFB No.: |  | EFB-C3a |
| --- | --- | --- |
| EFB Name.: |  | Seating at Commonly Accessible Routes |
| Category: |  | C. Well-being |
| Extent of Application: |  | Common Areas and Facilities |
| Recommended Design Requirements: |  | Resting places with resting facilities, such as seats (including fold-down seats) or lean on railings, should be: (i) provided on stair landings; (ii) provided in corridors more than 50 m in length; or (iii) provided in typical lift lobbies Resting facilities above should not reduce the statutory requirements on the clear width of access and manouvering space, and should not cause obstruction to access, circulation and exit routes. |
| Assessment Criteria: |  | - Resting facilities (e.g. permeant seats, folding seats, lean on railings, etc.) shall be provided along the major access route(s) at the following locations: - Staircase landings; or - Long corridors (exceeding 50 meters); or - Typical lift lobbies. - When identifying the major access route(s), the circulation paths accessing to all of the following space types from the main building entrance(s) shall be considered: - Main lobby/ foyer; - Lift lobbies; - Clubhouse entrance; - Shops, restaurants, cinemas, etc.; - Toilets; - External recreational facilities. - For each podium and block/tower (as applicable), accessible routes shall be provided from the main entrance to each of the required facility types listed in Criterion 3. These routes do not need to form a single continuous path; multiple distinct accessible routes may be used. However, there shall be at least one compliant accessible route to each listed facility type, ensuring that every required destination is reachable via a route that meets all relevant accessibility requirements. This is to provide safe and inclusive access, particularly for elderly individuals. - At least one seating facility shall be provided for every 50 meters at long corridors. - For interconnected spaces between staircases, long corridors, and lift lobbies, seating facilities shall be arranged to ensure the elderly do not need to walk more than 50 meters to reach a seating facility. - Back of house staircases are excluded in the assessment. |
| Submittals: | - EFB-C3a_01 Provide annotated layout drawing(s) to identify: - The major access route(s); and - The location(s) of the resting facilities provided. |  |
|  | - EFB-C3a_02 Provide drawing(s) and/ or sample photo(s) to demonstrate: - Resting facilities are provided; and - The resting facilities in a long corridor are spaced less than 50 meters apart (where applicable). |  |

Major access route is referred to the primary pathway or circulation route that enables safe, convenient, and unobstructed movement within the building that connects key areas such as entrances, elevators, staircases, communal spaces, and essential facilities, etc. For detailed step-by-step guidance on determining the major access route, refer to Appendix B1.

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.