| EFB No.: | EFB-C3b |
| --- | --- |
| EFB Name.: | Public Transport Interchange Seating |
| Category: | C. Well-being |
| Extent of Application: | Common Areas and Facilities |
| Recommended Design Requirements: | Long corridors and passageways within transport stations, interchanges and passenger terminals, and public passages within a building connecting with transport stations, interchanges and passenger terminals should be provided with resting places with resting facilities. The resting facilities should be provided in recessed areas at maximum intervals of 50 m. Resting facilities for the above should not reduce the statutory requirements on the clear width of access and manouvering space, and should not cause obstruction to access, circulation and exit routes. |
| Assessment Criteria: | - Resting facilities (e.g., permanent seats, folding seats, lean-on railings, etc.) shall be provided at all intersections between the building and transport stations, interchanges, and passenger terminals. |
| Submittals: | - EFB-C3b_01 Provide annotated layout drawing(s) to identify: - The location(s) of the resting facilities provided. |
|  | - EFB-C3b_02 Provide drawing(s) and/ or sample photo(s) to demonstrate: - Resting facilities are provided. |

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.