| EFB No.: | EFB-C4a |
| --- | --- |
| EFB Name.: | Elderly Friendly Toilets |
| Category: | C. Well-being |
| Extent of Application: | Common Areas and Facilities |
| Recommended Design Requirements: | Elderly-friendly toilets should be provided and located in an easily accessible common area or within a toilet with multiple cubicles. A sample of elderly-friendly toilet is shown below. Reference: Design Manual: Barrier Free Access 2008 (2025 Edition), Buildings Department, HKSAR |
| Assessment Criteria: | - At least one elderly-friendly toilet shall be provided in each group of toilets, provided it is accessible from a common area. Alternatively, at least one cubicle in each male and female toilet within each group of toilets shall be designated as elderly-friendly. - Each group of toilets is defined as a cluster of toilet facilities that are located together within a specific area and serve the needs of visitors in that section of the building. - Elderly-friendly toilets shall comply with the following requirements: - Open outward / folding door/ sliding door; - Grab bars on both sides of the cubicle, one side with L-shape grab bars; - Emergency call buttons complying with BFA’s Division 17 in the cubicle; - Hands-free sensor type toilet flush; - Holding device for walking sticks and bags; - Minimum internal width 900 mm and door width 850 mm; - Adequate manoeuvring space of 350 mm diameter; and - A sign installed outside showing the cubicle is equipped with elderly friendly facilities. - A unisex toilet or accessible toilet that complies with sub-items 3.1 to 3.8 above is regarded as meeting the requirement. |
| Submittals: | - EFB-C4a_01 Provide annotated layout drawing(s) to identify: - The location(s) of the elderly friendly toilets. |
|  | - EFB-C4a_02 Provide drawing(s) and/ or sample photo(s) to demonstrate: - Compliance with sub-items 3.1 to 3.8. |

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.