| EFB No.: | EFB-C5a |
| --- | --- |
| EFB Name.: | Elderly Friendly Changing Station |
| Category: | C. Well-being |
| Extent of Application: | Common Areas and Facilities |
| Recommended Design Requirements: | Elderly-friendly changing station/ facilities with an adult sized, height adjustable changing bench, either wall mounted or free standing in accessible toilets should be provided. |
| Assessment Criteria: | - An adult-sized, height-adjustable changing bench (either wall-mounted or free-standing) shall be provided in accessible toilets within each group of toilets. At least one cubicle in each male and female toilet within each group of toilets shall be equipped with a height-adjustable changing bench. - Each group of toilets is defined as a cluster of toilet facilities that are located together within a specific area and serve the needs of visitors in that section of the building. |
| Submittals: | - EFB-C5a_01 Provide annotated layout drawing(s) to identify: - The location(s) of the elderly friendly changing station. |
|  | - EFB-C5a_02 Provide drawing(s) and/ or sample photo(s) to demonstrate: - An adult-sized, height-adjustable changing bench is provided. |

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.