| EFB No.: | EFB-C8a |
| --- | --- |
| EFB Name.: | Community Planter Design |
| Category: | C. Well-being |
| Extent of Application: | Common External Recreation Spaces with Community Farming |
| Recommended Design Requirements: | Knee spaces should be provided under the planters for community farming which can help to improve focus, memory and cognitive function. A sample is shown below. Reference: Design Manual: Barrier Free Access 2008 (2025 Edition), Buildings Department, HKSAR |
| Assessment Criteria: | - Knee spaces shall be provided under the planters for community farming. - The planters shall have a maximum height of 750 mm measured from the top of the planter to the finished floor level. - Knee spaces shall be at least 680 mm measured from the bottom of the planter to the finished floor level. |
| Submittals: | - EFB-C8a_01 Provide annotated layout drawing(s) to identify: - The location(s) of the planters for community farming. |
|  | - EFB-C8a_02 Provide drawing(s) and/ or sample photo(s) to demonstrate: - The height of the planter is less than 750 mm. - The knee spaces underneath the planter is at least 680 mm. |

At least one set of photos shall be provided for each building in the project, with measurements of the relevant dimensional requirements clearly shown in the photos where applicable.